| Project Name | Kalgoorlie Airport Hotel (CDI 6425) | Location | 10 Gatacre Drive, Somerville, WA 6430 |
| --- | --- | --- | --- |
| Pour / lot ID |  | Pour date |  |
| Location / grids |  | Drawing no. & rev |  |
| Element | ☐ Pad ☐ Strip ☐ Slab ☐ Pit ☐ Kerb / path ☐ Other |  |  |
| Concrete Mix |  | Est. volume (m3) |  |
| Earthworks lot (ITR-001) |  | Cover required (mm) |  |

| No. | Question |  |  | Yes |  | No | N/A | Comment / result | Init. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BEFORE FORMING |  |  |  |  |  |  |  |  |  |
| 1 | Has the earthworks lot been handed over (ITR-001 signed), and is the base undisturbed since? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 2 | Has the set-out been checked against the current drawing revision? |  |  | ☐ |  | ☐ | ☐ |  |  |
| B. FORMWORK |  |  |  |  |  |  |  |  |  |
| 3 | Are the forms at the correct line, level and dimensions? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 4 | Are the forms braced, pinned and strong enough to hold the pour? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 5 | Are the forms clean, with joints sealed and release agent applied before the reo went in? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 6 | Are all rebates, set-downs, thickenings, blockouts and stop-ends in place? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 7 | Are 25 mm chamfers fitted where required? |  |  | ☐ |  | ☐ | ☐ |  |  |
| C. SAND BLINDING AND PLASTIC |  |  |  |  |  |  |  |  |  |
| 8 | Has the base been sand blinded, and is it smooth and free of sharp objects? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 9 | Is the plastic laid with 200 mm taped laps, turned up at the edges, with no tears or holes? |  |  | ☐ |  | ☐ | ☐ |  |  |
| D. REINFORCEMENT |  |  |  |  |  |  |  |  |  |
| 10 | Has the reo delivery been checked against the schedule and certificates? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 11 | Are the bar sizes, spacing and laps as shown on the drawings? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 12 | Is the cover correct? Required: ______ mm Measured: ______ mm |  |  | ☐ |  | ☐ | ☐ |  |  |
| 13 | Are chairs at the correct spacing and sitting on plates? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 14 | Is the reo tied, secure, and clean of mud, oil and loose rust? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 15 | Are all starter bars capped? |  |  | ☐ |  | ☐ | ☐ |  |  |
| E. CAST-INS |  |  |  |  |  |  |  |  |  |
| 16 | Have the other trades installed and signed off their cast-ins and penetrations? |  |  | ☐ |  | ☐ | ☐ |  |  |
| F. PRE POUR |  |  |  |  |  |  |  |  |  |
| 17 | Is the correct mix ordered, and are the tester, finishers, vibrators (plus a spare) and curing materials booked or on site? |  |  | ☐ |  | ☐ | ☐ |  |  |
| 18 | Has the weather been checked against the Hot Weather Concreting Plan? |  |  | ☐ |  | ☐ | ☐ |  |  |
| HP1 | HOLD POINT: Pre-pour release: no concrete is to be ordered until every party below has signed. |  |  |  |  |  |  |  |  |
| KBS Supervisor |  | Name / sign / date: | Electrical |  | Name / sign / date: |  |  |  |  |
| Hydraulic / Fire |  | Name / sign / date: | Other trade: ________ |  | Name / sign / date: |  |  |  |  |
| Structural Engineer |  | Name / sign / date: | CDI Site Manager (release) |  | Name / sign / date: |  |  |  |  |

| No. | Question | Yes | No | N/A | Comment / result | Init. |
| --- | --- | --- | --- | --- | --- | --- |
| G. DURING THE POUR |  |  |  |  |  |  |
| 19 | Was dockets checked for the correct mix, and were concrete temperature and time since batching within limits? | ☐ | ☐ | ☐ |  |  |
| 20 | Were slump tests done and cylinders taken as required? | ☐ | ☐ | ☐ |  |  |
| 21 | Were any non-conforming loads rejected and recorded? | ☐ | ☐ | ☐ |  |  |
| 22 | Was the concrete placed in layers and vibrated, with the forms watched throughout? | ☐ | ☐ | ☐ |  |  |
| 23 | Was evaporative retarder used where required, and curing started straight after finishing? Method / time: ______ | ☐ | ☐ | ☐ |  |  |
| H. POST POUR |  |  |  |  |  |  |
| 24 | Were control joints saw cut on the same day, to the correct depth? | ☐ | ☐ | ☐ |  |  |
| 25 | Was the formwork stripped without damage, and were tie holes filled? | ☐ | ☐ | ☐ |  |  |
| 26 | Is the finished surface free of honeycombing, cracking and other defects? | ☐ | ☐ | ☐ |  |  |
| 27 | Is the post-pour level survey within tolerance? | ☐ | ☐ | ☐ |  |  |
| 28 | Do the 7 and 28 day cylinder results pass? | ☐ | ☐ | ☐ |  |  |
| 29 | Have all defects been repaired and any NCRs closed out? | ☐ | ☐ | ☐ |  |  |
| HP2 | LOT CLOSED: Are all items complete, with records attached? KBS Supervisor: Name ________________ Sign __________ Date ________ CDI: Name ________________ Sign __________ Date ________ |  |  |  |  |  |

Comments: