| Project Name | Kalgoorlie Airport Hotel (CDI 6425) | Location | 10 Gatacre Drive, Somerville, WA 6430 |
| --- | --- | --- | --- |
| Lot / area ID |  | Date opened |  |
| Location / grids |  | Drawing no. & rev |  |
| Work type | ☐ Bulk platform ☐ Footings / Strips ☐ Lift pit ☐ Pavement subgrade ☐ Other |  |  |
| Design formation RL |  | Min. compaction | 95% under pads / strips |
| Subgrade CBR (C-010) | Min 5% In-situ result: ______ % | Improvement fill depth | Min 500 mm Placed: ______ mm |

| No. | Question | Yes | No | N/A | Comment / result | Init. |
| --- | --- | --- | --- | --- | --- | --- |
| A. BEFORE STARTING |  |  |  |  |  |  |
| 1 | Have all in ground services been located? | ☐ | ☐ | ☐ |  |  |
| 2 | Have dated photos been taken of the area and its surrounds before any work started? | ☐ | ☐ | ☐ |  |  |
| 3 | Are sediment controls and tree protection (by others) in place? | ☐ | ☐ | ☐ |  |  |
| 4 | Has the work area been set out from CDI's survey control? | ☐ | ☐ | ☐ |  |  |
| B. CLEARING AND STRIP |  |  |  |  |  |  |
| 5 | Have all vegetation, roots, rubbish and organic material been removed from the area? | ☐ | ☐ | ☐ |  |  |
| 6 | Has the topsoil been stripped and stockpiled separately for landscaping use only? | ☐ | ☐ | ☐ |  |  |
| 7 | Is the stripped surface free of soft spots, old fill and debris? | ☐ | ☐ | ☐ |  |  |
| 8 | If any unexpected material was found (unknown fill, contamination, suspected asbestos or rock), was work stopped and CDI notified? | ☐ | ☐ | ☐ |  |  |
| C. SUBGRADE AND FILL |  |  |  |  |  |  |
| 9 | Has the existing subgrade been compacted and tested? Nuke test Result ____ % (95% MMDD) or CBR Result: ______ % (5%) | ☐ | ☐ | ☐ |  |  |
| 10 | Where the CBR is below 5%, has at least 500 mm of approved fill been placed? Depth placed: ______ mm | ☐ | ☐ | ☐ |  |  |
| 11 | Has the fill source been approved, with test certificates received? | ☐ | ☐ | ☐ |  |  |
| 12 | Is the imported fill clean sand or cracker dust, free of clay, rubbish and stones over 75 mm? | ☐ | ☐ | ☐ |  |  |
| 13 | Were service trenches inspected, and as-constructed details received, before filling over them? | ☐ | ☐ | ☐ |  |  |
| 14 | Is the finished platform at design level and graded to drain, with no ponding? Survey ref: ______ | ☐ | ☐ | ☐ |  |  |
| HP1 | HOLD POINT: Has the subgrade been proof rolled with no deflection, heave or rutting, and has the geotech confirmed CBR of at least 5% after filling? Released by (Geotechnical engineer): Name ____________________ Sign ______________ Date / time ____________ |  |  |  |  |  |
| D. FOOTING AND PIT EXCAVATION |  |  |  |  |  |  |
| 15 | Have the footings and pits been excavated to the correct line, level and size? | ☐ | ☐ | ☐ |  |  |
| 16 | Has any over-excavation been recorded and made good? | ☐ | ☐ | ☐ |  |  |
| 17 | For excavations deeper than 1.5 m: are the sides battered, benched or shored, with edge protection and ladder access, and is the daily inspection signed? | ☐ | ☐ | ☐ |  |  |
| 18 | Are spoil and plant kept at least 1 m back from the excavation edge? | ☐ | ☐ | ☐ |  |  |
| 19 | Is the base clean, dry and free of loose material? | ☐ | ☐ | ☐ |  |  |
| 20 | Has the base under the footings been compacted and nuke tested to 95% modified? Result: ______ % | ☐ | ☐ | ☐ |  |  |
| HP2 | HOLD POINT: Has the founding been approved? Released by (Geotech / structural engineer): Name ____________________ Sign ______________ Date / time ____________ |  |  |  |  |  |
| HP3 | HANDED OVER TO CONCRETE: Is the area ready to hand over to concrete works? KBS Supervisor: Name ________________ Sign __________ Date ________ CDI: Name ________________ Sign __________ Date ________ |  |  |  |  |  |

Comments: record every 'No' answer here with the action taken, plus defects and photo refs